Interrupt Helper v1.0               H-09

INT 09 C - IRQ1 - KEYBOARD DATA READY

Desc:	this interrupt is generated when data is received from the keyboard.

This is normally a scan code (from either a keypress *or* a key

release), but may also be an ACK or NAK of a command on AT-class

keyboards.

Notes:	this IRQ may be masked by setting bit 1 on I/O port 21h

if the BIOS supports an enhanced (101/102-key) keyboard, it calls

INT 15/AH=4Fh after reading the scan code (see #0005) from the

keyboard and before further processing; all further processing uses

the scan code returned from INT 15/AH=4Fh

the default interrupt handler is at F000h:E987h in 100%-compatible

BIOSes

the interrupt handler performs the following actions for certain

special keystrokes:

Ctrl-Break	 clear keyboard buffer, place word 0000h in buffer,

invoke INT 1B, and set flag at 0040h:0071h

SysReq	 invoke INT 15/AH=85h (SysReq is often labeled SysRq)

Ctrl-Numlock place system in a tight wait loop until next INT 09

Ctrl-Alt-Del jump to BIOS startup code (either F000h:FFF0h or the

destination of the jump at that address)

Shift-PrtSc	 invoke INT 05

Ctrl-Alt-Plus (HP Vectra) enable keyclick

Ctrl-Alt-Plus (many clones) set clock speed to high

Ctrl-Alt-Minus (HP Vectra) reduce keyclick volume

Ctrl-Alt-Minus (many clones) set clock speed to low

Ctrl-Alt-SysReq (HP Vectra) generate hard reset

Ctrl-Alt-S	(many clones) run BIOS setup program

Ctrl-Alt-Esc (many clones) run BIOS setup program

Ctrl-Alt-Ins (many clones) run BIOS setup program

Ctrl-Alt-LeftShift-GrayMinus (some clones) turn off system cache

Ctrl-Alt-LeftShift-GrayPlus (some clones) turn on system cache

DR DOS hooks this interrupt to control the cursor shape (underscore/

half block) for overwrite/insert mode

DR Multiuser DOS hooks this interrupt for cursor shape control and to

control whether Ctrl-Alt-Del reboots the current session or the

entire system

SeeAlso: INT 05"PRINT SCREEN",INT 0B"HP 95LX",INT 15/AH=4Fh,INT 15/AH=85h

SeeAlso: INT 16/AH=00h,INT 16/AH=10h,INT 1B,INT 2F/AX=A901h,INT 4A/AH=00h"TI"

SeeAlso: INT 51"DESQview",INT 59"DoubleDOS",INT 79"GO32"

(Table 0005)

Values for keyboard make/break (scan) code:

01h	Esc		 31h	N

02h	1 !		 32h	M

03h	2 @		 33h	, <		 63h	F16

04h	3 #		 34h	. >		 64h	F17

05h	4 $		 35h	/ ?		 65h	F18

06h	5 %		 36h	Right Shift	 66h	F19

07h	6 ^		 37h	Grey*		 67h	F20

08h	7 &		 38h	Alt		 68h	F21 (Fn) [*]

09h	8 *		 39h	SpaceBar	 69h	F22

0Ah	9 (		 3Ah	CapsLock	 6Ah	F23

0Bh	0 )		 3Bh	F1		 6Bh	F24

0Ch	- _		 3Ch	F2		 6Ch	--

0Dh	= +		 3Dh	F3		 6Dh	EraseEOF

0Eh	Backspace	 3Eh	F4

0Fh	Tab		 3Fh	F5		 6Fh	Copy/Play

10h	Q		 40h	F6

11h	W		 41h	F7

12h	E		 42h	F8		 72h	CrSel

13h	R		 43h	F9		 73h	<delta> [*]

14h	T		 44h	F10		 74h	ExSel

15h	Y		 45h	NumLock		 75h	--

16h	U		 46h	ScrollLock	 76h	Clear

17h	I		 47h	Home

18h	O		 48h	UpArrow

19h	P		 49h	PgUp

1Ah	[ {		 4Ah	Grey-

1Bh	] }		 4Bh	LeftArrow

1Ch	Enter		 4Ch	Keypad 5

1Dh	Ctrl		 4Dh	RightArrow

1Eh	A		 4Eh	Grey+

1Fh	S		 4Fh	End

20h	D		 50h	DownArrow

21h	F		 51h	PgDn

22h	G		 52h	Ins

23h	H		 53h	Del

24h	J		 54h	SysReq		---non-key codes---

25h	K					 00h	kbd buffer full

26h	L		 56h	left \| (102-key)

27h	; :		 57h	F11		 AAh	self-test complete

28h	' "		 58h	F12		 E0h	prefix code

29h	` ~					 E1h	prefix code

2Ah	Left Shift	 5Ah	PA1		 EEh	ECHO

2Bh	\ |		 5Bh	F13 (LWin)	 F0h	prefix code (key break)

2Ch	Z		 5Ch	F14 (RWin)	 FAh	ACK

2Dh	X		 5Dh	F15 (Menu)	 FCh	diag failure (MF-kbd)

2Eh	C					 FDh	diag failure (AT-kbd)

2Fh	V					 FEh	RESEND

30h	B					 FFh	kbd error/buffer full

Notes:	scan codes 56h-E1h are only available on the extended (101/102-key)

keyboard and Host Connected (122-key) keyboard; scan codes 5Bh-5Dh

are only available on the 122-key keyboard and the Microsoft Natural

Keyboard; scan codes 5Eh-76h are only available on the 122-key

keyboard

in the default configuration, break codes are the make scan codes with

the high bit set; make codes 60h,61h,70h, etc. are not available

because the corresponding break codes conflict with prefix codes

(code 2Ah is available because the self-test result code AAh is only

sent on keyboard initialization).  An alternate keyboard

configuration can be enabled on AT and later systems with enhanced

keyboards, in which break codes are the same as make codes, but

prefixed with an F0h scan code

prefix code E0h indicates that the following make/break code is for a

"gray" duplicate to a key which existed on the original PC keyboard;

prefix code E1h indicates that the following make code has no

corresponding break code (currently only the Pause key generates no

break code)

the Microsoft Natural Keyboard sends make codes 5Bh, 5Ch, and 5Dh

(all with an E0h prefix) for the Left Windows, Right Windows, and

Menu keys on the bottom row

the German "Cherry G81-3000 SAD/04" keyboard contains contacts for

four additional keys, which can be made available by a user

modification; the three new keys located directly below the cursor

pad's Delete, End, and PgDn keys send make codes 66h-68h (F19-F21);

the fourth new key, named <delta>, sends make code 73h

the SysReq key is often labeled SysRq

the "Accord" ergonomic keyboard with optional touchpad (no other

identification visible on keyboard or in owner's booklet) has an

additional key above the Grey- key marked with a left-pointing

triangle and labeled "Fn" in the owner's booklet which returns

scan codes E0h 68h on make and E0h E8h on break

SeeAlso: #0499 at INT 16/AX=6F07h,#2469 at INT 4A/AH=05h